Faculty of Engineering, Architecture and Science

Department of Mechanical, Industrial, and Mechatronics Engineering

| Program | Electrical Engineering |
| --- | --- |
| Course Number | ELE888 |
| Course Title | Intelligent Systems |
| Semester/Year | W2026 |
| Instructor | Mohammed Saif |

| Lab/Tutorial Report NO. | Lab Report #4 |
| --- | --- |

| Report Title | Lab 4: Unsupervised Learning |
| --- | --- |

| Section No. | 01 |
| --- | --- |
| Submission Date | April 9th, 2026 |
| Due Date | April 9th, 2026 |

| Name | Student ID | Signature* |
| --- | --- | --- |
| Shai Mourad | 501170415 |  |

| TA Name | Signature* |
| --- | --- |
| Ammad | Syed ammad shah (check Q1 part b plot) |

*By signing above you attest that you have contributed to this submission and confirm that all

work you have contributed to this submission is your own work. Any suspicion of copying or

plagiarism in this work will result in an investigation of Academic Misconduct and may result

in a “0” on the work, an “F” in the course, or possibly more severe penalties, as well as a

Disciplinary Notice on your academic record under the Student Code of Academic Conduct,

which can be found online at:

https://www.torontomu.ca/senate/policies/pol60.pdf.

## Abstract

This lab implements the k-means clustering algorithm in order to identify dominant colors in an image. The data of pixels is represented in an RGB space and grouped into clusters using iterative optimization. Overall performance is evaluated using the error criterion and the Xie–Beni index. Observations show sensitivity to initialization and the number of clusters, presenting the great importance of unsupervised learning.

## Objectives

The experiment implements k-means clustering algorithms from scratch and then applies them to image pixel data for dominant color detection. Then, using the error criterion 𝐽, the performance is analyzed while clustering quality is observed using the Xie–Beni (XB) index. From the above methods, the effect of different initializations and number of clusters is studied.

## Theory

The main focus of this experiment is k-means clustering. This is an unsupervised learning algorithm that partitions data into c clusters by minimizing intra-cluster variance and iteratively updates cluster assignments and centroids until convergence [1].

J = j = 1ck = 1nj||xk - μj||2 [1] [Equation 1]

xk = data point

μj = the centroid of cluster 𝑗

nj = number of samples in cluster j

The algorithm completes the following:

- Initialize cluster centroids randomly

- Assign each data point to the nearest centroid

- Recompute centroids based on assigned points

- Repeat until convergence or maximum iterations [1]

In order to evaluate the clustering quality, the Xie–Beni index is applied:

XB(c) = k = 1Nj = 1cujk||xk - μj||N•mini≠j||μi - μj|| [1] [Equation 2]

The lower the XB value, the better the clustering with compact and well-separated clusters [1].

## Dataset

Using the image house.tiff, the dataset in this experiment, where each pixel represents a data sample. Image dimensions (MxN) are measured and each pixel is represented as a 3D vector:

pi = [ri , gi , bi ]T [1] [Equation 3]

ri , gi , bi represent red, green, and blue intensities respectively.

Then, the image is reshaped into the dataset:

X∈R(M×N)×3 [1] [Equation 4]

Rows of X correspond to a pixel in RGB space.

## Methodology

The experiment begins with loading the input image and converting it to a clustering-suitable dataset. Each pixel is represented as a 3D vector (corresponding to RGB values). Image size (equation 2) is reshaped into a dataset (equation 4 -  each row represents a pixel). A custom implementation of the k-means clustering algorithm is applied. This initializes cluster centroids while assigning each data point to the nearest centroid (Euclidean distance). Afterwards, the algorithm updates the centroids as the mean of the assigned points. The process is repeated until convergence/maximum numbers of iterations are reached.

The maximum number of clusters is c = 2, with chosen centroids ([0, 0, 0]) and ([255, 255, 255]). J, the error criterion, is defined as the sum of squared distances between data points and their respective cluster centroids. After each iteration, the error criterion is calculated and plotted to observe convergence while being visualized in RGB space and reconstructed into a labeled image.

Next, c is increased to 5. Two independent runs are executed with randomly initialized centroids to evaluate sensitivity to initialization. The output clusters are plotted and compared.

An XB index (Xie–Beni index) is computed for both clustering results. This index evaluates the quality of the clustering from intra-cluster compactness and inter-cluster separation. The superior solution is the one with the lower XB result.

## Results

Figure 1. Original “House” Image

Figure 2. Error Criterion (J) vs C = 2 iterations

Figure 3. Evolution of Cluster Means in RGB Space (c = 2)

From “Figure 3”, cluster means begin from extreme values (black and white) and then shift progressively to the dominant color regions in the image. The movement stabilizes after some iterations, showing the convergence of the k-means algorithm.

Figure 4. RGB Clusters for C = 2 iterations

Figure 5. Labeled image for C = 2 iterations

For c = 2, J decreases monotonically over the iterations that show convergence of the k-means algorithm. The final cluster has cluster centroids that stabilize after a few iterations. This is a demonstration of efficient clustering. “Figure 3.” shows the RGB scatter plot. This figure shows two distinct clusters corresponding to the dominant color groups in the given image. “Figure 4.” confirms a successful segmentation of the image into two major color regions.

Figure 6. Initial Means for Runs 1 and 2

When c is changed to 5, two independent runs show slightly different results since they each have various initial centroid selections. From “Figure 4.”, a variation in cluster distribution is observed, indicating that k-means is sensitive to initialization. It can also be seen that some clusters overlap or differ in compactness between runs.

Figure 7. Xie-Beni Index for Runs 1 and 2

“Figure 5.” presents the Xie–Beni index values for both runs 1 and 2. Run #1 produces a lower XB value, indicating better clustering performance with more compact and well-separated clusters.

## Discussion

From the observations and results, clustering is effective in identifying dominant colors in an image. When c = 2, there is a quick convergence with a linear segmentation. This is due to a simplified image with two dominant color groups.

However as the clusters, c, are increased to 5 the algorithm becomes more sensitive to initialization. Various initial centroid values result in differing final clustering solutions, which is evident from the variation in RGB plots. A limit in k-means is therefore observed: it may converge to local minima depending on the starting conditions.

The Xie–Beni index further supports this observation. Run #1, which has a lower XB value has better-defined clusters and smaller intra-cluster distances and larger inter-cluster separation. Not all clustering results are equally optimal, despite using the same amount of clusters and dataset.

Some clusters appear less meaningful visually, suggesting that the more amount of clusters present does not always make an improvement in interpretability. A proper c value is therefore extremely important and problem-dependent.

Overall, the experiment shows both the strengths and weaknesses/limitations of the k-means algorithm from simplicity and efficiency and initialization sensitivity and local minima.

## Conclusion

The k-means clustering algorithm was properly designed and applied to image data for dominant color detection. This algorithm quickly grouped pixel data into clusters and showed clear convergence behavior.

K-means performs well for simple clustering problems. Mostly in regards to a small cluster size. Although, the algorithm is sensitive to the choice of initial centroids, which can significantly impact the final clustering outcome. This was observed in the variation between different runs for ( c = 5 ).

From the Xie–Beni index, a very useful quantitative measurement of cluster quality is observed. This confirmed the lower XB values also resulted in better overall performance.

To conclude, k-means is a super powerful and efficient learning tool, but requires careful consideration in regards to the initialization and cluster number for best results.

## References

[1] M. Saif, “Lab 4: Unsupervised Learning,” ELE888 Intelligent Systems, Department of Mechanical, Industrial, and Mechatronics Engineering, Toronto Metropolitan University, Winter 2026.

## Code

import numpy as np import matplotlib.pyplot as plt import imageio.v2 as imageio from mpl_toolkits.mplot3d import Axes3D # ========================= # Load Image # ========================= I = imageio.imread('house.png') # <-- change filename if needed # Fix grayscale or RGBA images if len(I.shape) == 2: # Grayscale → convert to RGB I = np.stack((I,)*3, axis=-1) elif I.shape[2] == 4: # RGBA → remove alpha channel I = I[:, :, :3] plt.imshow(I) plt.title("Original Image") plt.axis('off') plt.show() M, N, _ = I.shape X = I.reshape(M*N, 3).astype(np.float64) # ========================= # K-MEANS FUNCTION # ========================= def kmeans(X, c, init_means, max_iter=50, tol=1e-3): means = init_means.copy() J_history = [] means_history = [means.copy()] # <-- ADDED for iteration in range(max_iter): # Assign clusters distances = np.linalg.norm(X[:, None] - means, axis=2) labels = np.argmin(distances, axis=1) # Compute error J J = 0 for j in range(c): cluster_points = X[labels == j] if len(cluster_points) > 0: J += np.sum((cluster_points - means[j])**2) J_history.append(J) # Update means new_means = np.zeros_like(means) for j in range(c): cluster_points = X[labels == j] if len(cluster_points) > 0: new_means[j] = np.mean(cluster_points, axis=0) else: new_means[j] = means[j] means_history.append(new_means.copy()) # <-- ADDED # Check convergence if np.linalg.norm(new_means - means) < tol: break means = new_means return means, labels, J_history, means_history # <-- ADDED # ========================= # Q1: c = 2 # ========================= print("\n===== Q1: c = 2 =====") # Manually chosen initial means init_means_q1 = np.array([ [0, 0, 0], [255, 255, 255] ], dtype=float) means_q1, labels_q1, J_q1, means_hist_q1 = kmeans(X, 2, init_means_q1) # <-- UPDATED # (a) Plot Error J plt.plot(J_q1) plt.title("Error J vs Iteration (c=2)") plt.xlabel("Iteration") plt.ylabel("J") plt.show() # (b) Print means print("Initial Means:\n", init_means_q1) print("Final Means:\n", means_q1) # (b) Plot means evolution <-- ADDED fig = plt.figure() ax = fig.add_subplot(111, projection='3d') # initial, middle, final stages stages = [0, len(means_hist_q1)//2, -1] colors_stage = ['red', 'blue', 'green'] for i, stage in enumerate(stages): m = means_hist_q1[stage] ax.scatter(m[:, 0], m[:, 1], m[:, 2], c=colors_stage[i], s=100, label=f"Stage {stage}") ax.set_xlabel('R') ax.set_ylabel('G') ax.set_zlabel('B') plt.title("Cluster Means Evolution (c=2)") plt.legend() plt.show() # (c) RGB Space Plot fig = plt.figure() ax = fig.add_subplot(111, projection='3d') colors = means_q1[labels_q1] / 255 ax.scatter(X[:, 0], X[:, 1], X[:, 2], c=colors, s=1) ax.set_xlabel('R') ax.set_ylabel('G') ax.set_zlabel('B') plt.title("RGB Clusters (c=2)") plt.show() # (d) Labeled Image X_labeled_q1 = means_q1[labels_q1] I_labeled_q1 = X_labeled_q1.reshape(M, N, 3) plt.imshow(I_labeled_q1.astype(np.uint8)) plt.title("Labeled Image (c=2)") plt.axis('off') plt.show() # ========================= # Q2: c = 5 (Two Runs) # ========================= print("\n===== Q2: c = 5 =====") # Run 1 init1 = X[np.random.choice(len(X), 5, replace=False)] means1, labels1, _, _ = kmeans(X, 5, init1) # <-- UPDATED # Run 2 init2 = X[np.random.choice(len(X), 5, replace=False)] means2, labels2, _, _ = kmeans(X, 5, init2) # <-- UPDATED def plot_clusters(X, labels, means, title): fig = plt.figure() ax = fig.add_subplot(111, projection='3d') colors = means[labels] / 255 ax.scatter(X[:, 0], X[:, 1], X[:, 2], c=colors, s=1) ax.set_title(title) plt.show() plot_clusters(X, labels1, means1, "Run 1 (c=5)") plot_clusters(X, labels2, means2, "Run 2 (c=5)") print("Initial Means Run 1:\n", init1) print("Initial Means Run 2:\n", init2) # ========================= # Q3: Xie-Beni Index # ========================= print("\n===== Q3: Xie-Beni Index =====") def xie_beni(X, labels, means): N = len(X) c = len(means) numerator = 0 for i in range(N): numerator += np.linalg.norm(X[i] - means[labels[i]]) min_dist = np.inf for i in range(c): for j in range(c): if i != j: dist = np.linalg.norm(means[i] - means[j]) if dist < min_dist: min_dist = dist return numerator / (N * min_dist) xb1 = xie_beni(X, labels1, means1) xb2 = xie_beni(X, labels2, means2) print("XB Index Run 1:", xb1) print("XB Index Run 2:", xb2) if xb1 < xb2: print("Run 1 is better (lower XB index)") else: print("Run 2 is better (lower XB index)") # ========================= # END # =========================